Lab Session 12

High Level Language Interface

# Objectives

- General Conventions, Model Directive

- Implementing Inline Assembly Code

# .Model Directive

.MODEL directive determines

- memory model type

- procedure naming scheme

- parameter passing convention

Memory Model

Memory Options

Language specifier -> determines calling and naming conventions for procedures and public symbols

Stack distance -> can be NEARSTACK (the default) or FARSTACK

We mostly uses     .model flat, STDCALL

STDCALL is the language specifier used when calling MS-Windows functions.

# Steps to follow

1.

Select

New Project >

Console Application

2

. Build the example Code.

- Add Source File . Utility >> .txt file. Change the name and extension to .asm.

- Right Click Project >> Build Dependencies >> Build Customization >> Select MASM.

- Right Click on .asm File.>> Properties >> General >> Item Type >> MASM.

- Copy C++ code in .cpp file.

- Copy Assembly Code in .asm file.

- Build solution and observe result.

# Example:

# C++ Code:

#include <stdio.h>

// extern "C" instruct the compiler to use C calling conventions extern "C" void clear(); int main()

{

clear();

//define variables

unsigned short src_opnd, dst_opnd, src_rslt, dst_rslt;  	printf("Enter two 4-digit hex numbers - src, dst: \n");

scanf_s("%hX %hX", &src_opnd, &dst_opnd); // in scanf_s it is necessary to

//specifiy length

//switch to assembly

_asm

{

MOV AX, src_opnd

MOV BX, dst_opnd

SHRD BX, AX, 10; shift AX : BX right 10 bits

MOV src_rslt, AX

MOV dst_rslt, BX

}

printf("\nSource result = %X\n Destination result = %X\n\n", src_rslt, dst_rslt);  	return 0;

}

# Assembly Code:

.686   	 	 	;Target processor. Use instructions for Pentium class machines

.MODEL FLAT, C  	 	;Use the flat memory model. Use C calling conventions

.STACK 2048   	 	;Define a stack segment of 1KB (Not required for this example)

.DATA   	 	 	;Create a near data segment. Local variables are declared after

;this directive (Not required for this example)  var_1 dword 10

str_1 byte 50,100,34,5,6,78,12,45,67  str_2 byte 5000 dup(?)

.CODE   	 	 	;Indicates the start of a code segment. clear PROC   xor eax, eax   xor ebx, ebx   ret  clear ENDP  END

# Output:

ACTIVITIES:

- Write a program in C++ which takes input from user and contains a procedure in assembly named ThreeProd that displays the product of three numeric parameters passed in a function argument.

- Write a program in C++ which takes input from user and contains a procedure in assembly named GCD(Greatest common divisor) which calculates their GCD.